Activity 2: Worked answers

# Practice question 1

Step 1: Define the variables.

Remember, the base of this house is a rectangle.

- Let w be the width of the warehouse (in metres).

- The length is twice the width, so the length is 2w (in metres).

- Let h be the height of the warehouse (in metres).

The volume, V, of the warehouse is 2400 m3 so:

V=length ×width ×height

2400=w ×2w ×h

2w2h=2400

Solve for h:

h=24002w2=1200w2

Step 2: Set up the heat loss equations.

Heat loss through the walls

There are two walls of size wh, and two walls of size 2w × h.

The total wall area, Awalls, in terms of w is:

Awalls=2wh+2(2w×h)

Awalls=2wh+4wh

Awalls=6wh

Substitute h=1200w2 into the equation:

Awalls=6w1200w2

Awalls=7200w

The heat loss through the walls is proportional to the wall area.

Using the heat loss equation for the wall:

Qwalls= Uwalls 7200w∆T

Heat loss through the roof

The area of the roof (length × width) is Aroof=2w×w=2w2

We are told that twice as much heat per square metre is lost through the roof as through the walls. So know that Uroof=2Uwalls

Therefore, the heat loss through the roof is:

Qroof= Uroof 2w2∆T

Qroof= 2Uwalls 2w2∆T

Qroof= 4Uwalls w2∆T

Step 3: Combine the heat loss equations to calculate total heat loss.

Qw=Qwalls+Qroof

Qw=Uwalls 7200w∆T+4Uwalls w2∆T

Take out the common factor Uwalls∆T:

Qw=Uwalls ∆T7200w+4w2

Step 4: Use differentiation to find the point at which heat loss is a minimum.

To minimise the heat loss, we need to differentiate the total heat loss function Q (w) with respect to w and set the derivative equal to 0.

We can ignore the constant Uwalls∆T since it does not affect the minimisation. 
Rewrite the function as

qw=7200w+4w2

Then rewrite this with a negative power and differentiate:

qw=7200w-1+4w2

dqdw= - 7200w-2+8w

dqdw= - 7200w2+8w

Step 5: Set the derivative equal to 0 to find the critical points, and solve the equation.

dqdw= 0

So:	- 7200w2+8w=0

Rearrange the equation to make the terms positive and solve:

8w=7200w2

8w3=7200

w3=72008=900

w =3900=9.65 m

Step 6: Find the length and height.

The length is twice the width, so length = 2w=2 ×9.65=19.3 m

Use the formula for h to find the height:

h=1200w2=12009.652=12.89 m

Step 7: Verify it’s a minimum.

To verify that this is a minimum, check the second derivative of q(w).

dqdw= - 7200w-2+8w

d2 Qdw2=-2×-7200w-3+8

d2 Qdw2=2 7200w3+8

d2 Qdw2= 14400w3+8

Substitute in w = 9.65

14400(9.65)3+8 =  14400898.632+8 = 16+8=24

Since 24 > 0, the second derivative is positive, confirming that w=9.65 gives a minimum.

Final answer

The dimensions of the warehouse that minimise heat loss are approximately:

- Width = 9.65 m

- Length = 19.3 m

- Height = 12.89 m

# Practice question 2 – fully worked example

Step 1: Define the variables.

- Let w be the width of the storage unit in metres.

- The length is three times the width, so the length is 3w (in metres).

- Let h be the height of the storage unit in metres.

The volume of the storage unit is 3600 m3 so:

V=length ×width ×height

3600=w ×3w ×h

3w2h=3600

Solve for h:

h=36003w2=1200w2

Step 2: Set up the heat loss equations.

Heat loss through the walls

There are two walls of size wh, and two walls of size 3wh.

The total wall area is:

Areawalls=2×wh+2×3wh=2wh+6wh=8wh

Substitute h=1200w2 into the equation:

Awalls=8w1200w2

Awalls=9600w

The heat loss through the walls is proportional to the wall area. Use the heat loss equation for the wall:

Qwalls=Uwalls9600w∆T

Heat loss through the roof

The area of the roof = 3w ×w = 3w2

We know that 50% more heat per square metre is lost through the roof as through the walls. This means we know that Uroof=1.5Uwalls

Use the heat loss equation for the roof:

Qroof=Uroof3w2∆T

Qroof=1.5Uwalls3w2∆T

Qroof=4.5Uwallsw2∆T

Step 3: Combine the heat loss equations to calculate total heat loss.

The total heat loss Q is the sum of the heat loss through the walls and the roof:

Qw=Qwalls+Qroof

Qw=Uwalls 9600w∆T+4.5Uwalls w2∆T

Take out the common factor Uwalls∆T:

Qw=Uwalls ∆T9600w+4.5w2

Step 4: Use differentiation to find the point at which heat loss is at a minimum.

To minimise the heat loss, we need to differentiate the total heat loss function Q(w) with respect to w and set the derivative equal to 0.

We can ignore the constant Uwalls∆T since it does not affect the minimisation.

Rewrite the function as:

qw=9600w+4.5w2

Then rewrite this with a negative power and differentiate:

qw=9600w-1+4.5w2

dqdw= - 9600w-2+9w

dqdw= - 9600w2+9w

Step 5: Set the derivative equal to zero to find the critical points, and solve the equation.

dqdw= 0

So:	- 9600w2+9w=0

Rearrange the equation to make the terms positive and solve:

9w=9600w2

9w3=9600

w3=96009=1066.6667

w =31066.6667=10.2 m

Step 6: Find the length and height.

Now that we know the width, we can find the length and height.

The length is three times the width, so:

Length = 3w = 3 ×10.2 = 30.6 m

The height is given by:

h=1200w2

h=120010.22=11.5 m

Step 7: Verify it’s a minimum.

To verify that this is a minimum, check the second derivative of q(w).

dqdw= - 9600w-2+9w

d2 Qdw2=-2×-9600w-3+9

d2 Qdw2=2 9600w3+9

d2 Qdw2= 19 200w3+9

Substitute in w = 10.2

19 600(10.2)3+9 =  19 6001061.208+9 = 18.5+9=27.5

Since 27.5 > 0, the second derivative is positive, confirming that w = 10.2 gives a minimum.

Final answer

The dimensions of the storage unit that minimise heat loss are approximately:

- Width = 10.2 m

- Length = 30.6 m

- Height = 11.5 m